MODBUS MONITOR User Guide // v3.04

Overview

Modbus Monitor is a desktop application for real-time acquisition and logging of flow meter data over Modbus RTU (RS-485). It combines a live ZMQ subscriber for instantaneous flow rate display with a configurable Modbus register reader, and exports results to Excel for analysis or reporting.

ℹ NOTE: The flow rate display receives data from a Frog monitoring process via ZMQ on 127.0.0.1:5555. Frog must be running before starting this application.

Interface

Figure 1 — Main application window

Quick Start

| # | ACTION | DESCRIPTION |
| --- | --- | --- |
| 1 | Select COM Port | Choose the correct serial port from the dropdown list and click Connect. The port corresponds to the RS-485 adapter connected to your flow meter. |
| 2 | Add Device | Enter a device name in the Device field and the Modbus register address in the Address field, then click Add. A new row will appear in the data table. |
| 3 | Read Registers | Click Read to perform a single Modbus read cycle. The Read column will update with the current register value decoded as a 32-bit float. |
| 4 | Auto Update | Enable Auto Update to continuously read all registers every second. The button will highlight green when active. Click again to stop. |
| 5 | Auto Test | Use Auto Test (60 s) to automatically save a snapshot of all readings to the Excel file every 60 seconds — useful for acceptance testing. |
| 6 | Save / Load | Use Save File to export the current table to example.xlsx, or Load File to restore a previously saved session. |

Register Encoding

Flow values are stored as IEEE 754 single-precision floats across two consecutive 16-bit Modbus holding registers in Little-Endian Word Order (low word first):

| Register | Content | Notes |
| --- | --- | --- |
| addr + 0 | Low word (LSW) | Bits 0–15 of the float32 |
| addr + 1 | High word (MSW) | Bits 16–31 of the float32 |

Requirements

| DEPENDENCY | DETAILS |
| --- | --- |
| Python | 3.10+ |
| PySide6 | Qt 6 bindings for the GUI layer |
| pymodbus | Modbus RTU client over serial |
| pyzmq | ZMQ subscriber for live flow data |
| pandas / openpyxl | Excel read/write support |
| Frog Monitor | Must be running and publishing on tcp://127.0.0.1:5555 |

Modbus Monitor  //  User Guide  //  v3.04  //  2026